Georgi Dimantchev

THE MOST IMPORTANT ABOUT MY COMPOSITE MTA BOOMERANGS

"Gradus", "Sensor" and "Primer"

I have been making and throwing boomerangs since 1970, MTA boomerangs since

1992 and also Composite boomerangs since 1992. Therefore one can say that my

latest Composite MTA boomerangs - "Gradus", "Sensor" and "Primer", are the

result of many years of investigation and experience. Step by step I have

always sought the best solutions to theoretic and practical problems -

principally in two directions - aerodynamics and composite construction

technology.

Aerodynamic Design

In aerodynamic design I have experimented with the outline shape and also

with the arm profile. The shapes of all three MTA models appear so simple

and familiar. But they were developed and optimised after much time in the

building and testing of many prototypes. During the tests many MTAs (at

least 10!) - prototypes and Composite copies - were lost into the blue! Two

important steps on the way to the Composite MTAs production were:

1. The series of "Magic", "Topic" and "Impulse" MTA boomerangs made from

1.55 and 2.00 mm thick fiberglass/epoxy/metal-plates (1993-95) and

2. The moulded in matrix Composite "Vector" MTA boomerang (1995-96).

I have already published my ideas in the area of boomerang arm profiles in

the "Swiss Boomerang Newsletter" (No. 1 & 2/2000). My theory indicates that

it is better for both arms of an MTA boomerang to have different profiles.

In the models "Gradus" and "Sensor 2" the leading arm has a slightly curved

(cambered) profile underneath. Earlier Wilhelm Bretfeld and Adam Mueller

(Germany) had already experimented with hollowed underside of MTA

boomerangs. I decided to use new and different profiles for the both arms

just after the effect was theoretically proven. Very encouraged for me were

the throwing tests' results of "Gradient" MTA (developed in

November-December 1999; span 330 mm, thickness 3.2 mm, mass 18 g; wich plan

was published in Swiss, Australian, Holland, French, Italian and British

boomerang newsletters in 1999-2000). After many 1+ min. flights the

"Gradient" second prototype was lost from the sigh after 2 min. 30 sec.

"time aloft" on the 03.12.1999. One of the next 5 made of plywood "Gradus"

prototypes was selected for series production on the 26.03.2000.

Although the theory was clear, the optimal arm profile was long sought after

- for the "Gradus" series alone three matrixes were made, whereby the arm

profile thickness and camber were gradually optimised. Now in the series

production is only "Gradus 2" with double reduced LA-profile' camber. The

next model - "Sensor" has the outline shape and tuning from Manuel Schuetz's

"Apollo 3" (ex-Swiss MTA100 record - 55.52 sec., 27.07.1996, Bensheim,

Germany), its profile was modified right from the beginning and "Sensor" is

today produced in two variations (in two matrixes) in series:

- "Sensor 1" with flat LA-underside and

- "Sensor 2" with cambered LA/elbow-underside.

Of decisive importance to the flight-path (flight distance, height and

hover) and also for wind-resistance is the surface loading (boomerang'

mass-to-circle'area ratio). The given on the picture with the outline shapes

tuning data (bending and twisting) were optimised over a long time in close

connection with the surface loading. (Because Composite technology makes it

possible to make an MTA boomerang between 10 and 50 g in weight. Where in

between does the optimum lie?). For stable hover the models "Gradus" and

"Primer" are sensitively balanced with two micro weights - 0.4-0.5 g in

weight, built in to trailing arm.

The Composite technology was tested and optimised over many years by the

design and production in series of 6 different boomerang models:

- "Classic" (1991-92, span 395 mm, mass 68 g, 120 copies made),

- "Weekend" Hook (1992, span 318 mm, mass 50 g, 8 made),

- "Hope" V/Omega (1992, span 363 mm, mass 62 g, 8 made),

- "Vector" MTA (1995-96, span 310 mm, mass 28-30 g, 80+ made),

- "Legend" Hook (1991-92, span 305 mm, mass 52 g, 6 made) and

- "Aussie Stick" Kylie (1992/2000, span 560 mm, mass 350 g, 20 made).

The first matrix for the "Gradus" was ready on 03.04.2000, the first

Composite "Gradus" prototype was built on 06.06.2000, "Gradus 2" on

25.10.01, "Gradus 1M" on 05.12.2001, "Sensor" on 22.07.2001, "Sensor 2" in

September 2001 and "Primer" on 02.11.2001.

Many boomerang designers/throwers from quite different corners of the world

are interested in my Composite MTA boomerangs. They also wished to know the

differences in comparison with the "Jonas Composite" - the model of the

pioneer in Composite MTA manufacture - the Swede Jonas Romblad (1991, "Wind

Sailor" shape; 1 min. 37.78 sec. - European MTA100 record by David Hesse,

20.09.1997, Burgdorf, Switzerland). I think, the aerodynamic and technology

level of my Composites are at least a couple of steps foreward on the

eternal way towards the optimum, but each thrower can decide himself on the

result. I didn't keep the construction a secret.

Construction, Materials

Each of my Composite MTA boomerangs consists of two shells. Each shell

consists of three to five layers and is made in especially for each of these

shell matrix from different reinforcement-materials and epoxy resin. For the

reinforcement are used products (fabric and/or rowing) of glass, carbon and

aramid ("Kevlar" TM by DuPont). The kind of each material and its order in

the shell are selected so that each materials give of their best qualities

for the strength and stiffness of the boomerang. The space between the two

shells is filled with a lightweight foam material, not like the "Jonas

Composite" which uses micro-balloons/epoxy mixture. I use materials that

come from Germany, France, Russia and the USA. All they are special

materials that were developed for building aircraft, sailplanes and

aero-models and are of the highest quality. In production some quite

specialised and relatively expensive tools were used, shears for cutting

aramid, cutters for carbon, scales with measuring accuracy of 0.1 g, vacuum

pump, special oven (autoclave) and more. After the closing of the two shells

the boomerangs were "baked" in the oven. Next the boomerangs were left to

"relax" from the high temperatures. Later the boomerangs were tested many

times in different atmospheric conditions (air-temperature, dryness and

humidity, dynamic and thermal air conditions) and over different times of

day. During the tests the boomerangs were tuned using "3D/Zig-Zag"

turbulators (already proposed by me to the boomerang world for 5 years).

With the relatively thin "Primer" it wasn't really necessary to use the

turbulators, but with the thicker "Gradus" and "Sensor" it was. To place the

turbulators right it was neccessery to investigate in advance the air-stream

distribution over the boomerang surfaces. This experiment was started in

June 2000 and  continues now. The picture of the air-stream for the all most

popular boomerang shapes - Hockey Stick, V/Omega and Hook were already "got"

in the summer of 2000.

Benefits

My Composite MTA boomerangs posses two most important qualities for every

one boomerang thrower:

1. They are tuned and retain the tuning durably - so that one doesn't have

to bend up or to twist the arms.

2. Throwing is relatively simple and with almost perfect classical style,

that is one holds the boomerang vertical and throws it about 45 up to 60

degrees over the horizon.

Care and Storage

For storage, transportation and throwing one must bear in mind that the

Composite MTA boomerang (as with all composite construction) is not

absolutely unbreakable and that they are sensitive to large temperature

variations. Therefore it is recommended that the Composite MTA boomerang is

transported and stored in a secure box and should only be thrown in a big

enough grass field.

What is typical for each of the three Composite MTA models:

- "Gradus" (span 333 mm, thickness 3.2 mm, standard-mass 19-21 g) because of

its dimensions and tuning it is most stable in wind. Andy Cross (Calgary,

Canada) wrote me "I really do like your "Gradus" MTAs. I think that they are

one of the best, if not the best, MTAs currently available." (28.02.2002);

- "Sensor"  (span 363 mm, thickness 3.2 mm, standard-mass 20-22 g, for

throwing in higher wind up to 25-26 g) it most closely resembles the

classical plywood MTA well liked by some throwers, and in part also to the

"Jonas Composite". Ted Bailey (Ann Arbor, Mi., USA) share me "I like that

model better than another similar Composite MTAs." (15.10.2001);

- "Primer" (span 340 mm, thickness 2.5 mm, standard-mass 18-19 g) is the

easiest for throwing and because of its low weight recommended for not too

strong wind. Specialy for throwing in high wind is produced "Primer Heavy"

with mass of 27-28 g. Didier Bonin (Quetigny, France) comented "... I tested

it comparing with the "Jonas Composite" and I can tell you this: the

"Primer" is as good as the "Jonas", as long in the air! Very very slow going

down!... Also the big point is: when you must difficulty throw the "Jonas"

negativ, you can throw the "Primer" easily more!!... Today was windy and it

was not a problem for the "Primer"!!" (20.11.2001).

Results

After so many words on design and technology, a little about experience.

What has been achieved in competition with my Composite MTA boomerangs?

- With "Vector" Michael "Gel" Girvin (Ca., USA) threw 43.19 sec. and won the

MTA event in a tournament in Allentown, Pennsylvania (summer 1997).

- Barnaby Ruhe (Pa., USA) wrote me "I won a few contests with one of your

"Vector" MTA's for big wind, until it flew away." (24.12.2001).

- One of the first "Gradus" thrown by me in the German Championship in

Muenster (June 2000) stayed in the air for 46.13 sec. and that was good

enough for only fifth place in the MTA event, but it was the new Bulgarian

MTA100 record.

- Using a "Gradus" (made in 2000), the American Lee Ragsdale won the MTA

event with three throws of 38, 46 and 39 sec. in the Georgia Tournament

(USA, April 2001).

- With a "Gradus" (made in 2001), the Englishman Andrew Furniss set the

British MTA Unlimited record with 75.41 sec. in the Trefriw Festival (UK,

August 2001).

As I have already said, Jonas Romblad started making his Composite MTA

boomerang in 1991. Gary Broadbent (Oh., USA) began first to copy the "Jonas"

("Bonas", since 1998). Harald Gerbig (Bavaria, Germany) was the second who

tried to copy the "Jonas" ("Pemulwuy", 1999). In the year 2000 Andy Cross

(Calgary, Canada), in 2001 also Manuel Schuetz (Gruenenmat, Switzerland) and

Martin Laslett (Birmingham, England) started making MTA boomerangs in

Composite construction. Manuel is to date the leader - with one of his

Composites ("Wind Sailor" shape) he has, with 50.06 sec. and 99.94 sec.

respectively, won the MTA100 and MTA100/3+ events in the 15th Swiss

Championship in Burgdorf in the summer of 2001.

A new era has started in MTA boomerang technology and a competition already

exists. That pleases me personally. A creative competition is very healthy.

Only in that way can each thrower choose the best for himself MTA boomerang

- moulded in matrix or shaped of plywood, paper-phenolic, linen-phenolic or

carbon/epoxy-plate. And the best should win!

Parallel with the production of the three MTA models, the Composite "Aussie

Stick" was produced. This is a true-to-the-original copy of an Australian

Kylie (Perth Region, W.A.) from the first half of the twentieth century. The

first "Aussie Stick" prototype was made in winter of 1992. The construction

has been optimised for a long time and now each "Stick" is unique. My

Composite Kylie is relatively light - it weighs "only" 350 g, it is however

strong enough - as an "original" from acacia (Mulga) wood and flew 80 and

more meters in a steep straight flight. As with my Composite MTA's, the

"Aussie Stick" consists of 2 shells, each shell consists however of more

vacuum pressed layers - up to 10(!) of which the proportion of E-glass is

the largest. The two upper layers are formed from carbon (black) and aramid

(yellow or light brown), which is similar to the arrangement of the plies in

the Mulga wood.

And what will be the next step? On the 11.11.2001 inspired by Manuel Schuetz

I made the first prototype of the "Trigger" - a triblader with typical for

one MTA boomerang flight characteristics. Up today were made 30 prototypes

of different materials and resp. with different thicknesses and weights. The

"Trigger"-shape is "creazy", but the profiling of it's three arm is "total

creazy" and this is the "little secret". With the last two "Trigger" I have

had already several times 30+ sec. (up to 33.62 sec.) "times aloft" in

middle wind. Very soon and for sure before the World Boomerang Championship

in Kiel, Germany (July/August 2002) will be started the Composite "Trigger"

MTA production. I hope this model will be the sure choice for the

"Supercatch" event when the wind is high.

PS. The "Trigger #1" was ready on the 03.05.2002. The first three Composite

"Trigger" prototypes were tested on the 10.05.2002. On the 13.05.2002, in

light wind and light thermic the "Trigger #2" stayed 47.37 sec. aloft and

land in the 100-m-circle. During the later tests I have with this sample as

best 43.18 and 39.08 sec.

PPS. For the series-"Trigger" is used new epoxy resine (German production

special for aircraft manufacturing) and new filling material. During the

first tests the "Trigger #7" (yellow/black surfaces, no weight-ballance)

hovered 30.43 and 32.25 s. On the 09.06.2002, in light wind and thermic with

the "Trigger #3" were thrown 33.19 and 36.68 s. and with the "#7" - 32.25 s.

(10.06.2002)

Finally, I would like to say "Thank you very much, dear friend!" to:

1. Manuel Schuetz, Didier Bonin, Ted Bailey, Pierre Kutek (France) and

Oliver Thienhaus (Germany) for their critical comments after testing my

Composite MTA boomerangs and

2. Andy Cross ("Boomerang News"/Canada), Ted Bailey ("Many Happy

Returns"/USA), Renan Guillou ("FBA Newsletter"/France), Martin Laslett ("BBS

Journal") and Maurizio Saba ("CLUB"/Italy) for publishing an information

about my Composite boomerangs.

Summarised: 15.12.2001-20.01.2002, Sofia, Bulgaria

Added news for the publication in "BBS Journal": 06.-08.03.2002

Published already:

1. In French in "FBA Newsletter", No.75/December 2001, pages 12-13

2. In German in "Swiss Boomerang Newsletter", No.4/Winter 2001, pages 11-13

3. In Italian in "CLUB" Newsletter, No.36/Febbraio 2002, pages 10-18

4. In English in "BBS Journal", Nr.21/Spring 2002, pages 18-22

Drawings and pictures:

1. "Impulse" MTA plan

2. "Gradient" MTA plan

3. The four GD' Composite MTA boomerangs: "Vector", "Gradus", "Sensor" and

"Primer"

4. Current GD' Composite boomerang production: "Gradus", "Sensor" and

"Primer"

5. "Jonas Composite" and "Gradus" MTAs

6. "Jonas Composite", "Manuel Composite", "Sensor" and "Apollo 3" MTAs

7. Tuning data (arm bending and twisting), CG position, thickness, mass of

the Composite "Gradus", "Sensor" and "Primer" MTA boomerangs

8. Arm profile and arm-thickness-to-width ratio of the GD Composite MTAs

9. Air-stream picture on the upper surface of "Zepter" and "Gradus" MTAs

10. Air-stream picture on the upper surface of "Vitamin" V/Omega

11. A girl with three "Gradus" MTAs in the hand

12. GD with an "Aussie Stick" in the hand (Muenster, Germany, June 2000)